Health and Safety at Work

Folder to be kept on site

In the Cleaners Cupboard

Health and Safety

Working Safely

Everyone at work, regardless of position or hours of work, has a duty to protect the health and safety of themselves and those around them.

This is a legal requirement of the Health and Safety at Work Act of 1974

Under the terms of this Act Eco-Clean has a legal requirement to:

- Provide and maintain safe plant, equipment and systems

- To ensure, as far as possible, safety and absence of risk to health in connection with the use, handling, storage and transport of articles and substances

- To provide such information, training, instruction and supervision as necessary to maintain health and safety standards

- To maintain safe places of work

- To provide a working environment which is as far as possible, safe and without risk to health with adequate regard to facilities and arrangements for welfare

Under the terms of this act Employees have a legal duty to:

- Take reasonable care for the health and safety of themselves and others who may be affected by their acts or omissions at work

- Co-operate with their employer so as to enable the employer to comply with the duties imposed, i.e. the employee must strictly obey the rules of all safe working procedures instigated by the employer.

- Not misuse anything provided in the interest of health and safety, any machinery, chemical substances or equipment and to report any defects they are aware of.

REMEMBER

- Always follow your cleaning schedules and any instructions or guidelines you have been given

- Read the Risk Assessments associated to your task

- Never take or create a risk

- Always use your personal protective equipment correctly

Your Health and Safety Representative Is Daniel Gibson, who can be contacted on 07581154818

## Quality Policy

We strive to provide a service of exceptional quality and are committed to achieving this through the following methods:

- Continually train, support and motivate all staff, operative and management.

- Maintain a controlled, documented system of our performance.

- Log and analyse any complaints or faults and take the appropriate corrective action to ensure continuous improvements in the quality of service to our customers.

- Execute audits of the quality system to ensure continuous compliance with this policy statement.

- Review the success of this policy on a regular basis.

## Environmental Policy

Eco-clean is a company that cares about the environment. The majority of our cleaning products we use are biodegradable and environmentally friendly, and all our literature is made from recycled paper.

Telephone:  01926 915390   Mobile:  07581154818

Email: Daniel@ecocleanservice.co.uk

Registered in England No 6206500

Unit 2, Whitehouse Farm, Oxford Road

Princethorpe, Rugby CV32 9QD

STAFF CONFIDENTIALITY AGREEMENT

To acknowledge personal responsibility concerning security and confidentiality of information (relating to yourself as a contracted cleaning client)

I acknowledge that during my cleaning employment, any confidential information must not be disclosed.

I agree that this confidentiality is applied  during  my  working relationship with       Eco-Clean & Maintenance Ltd and after the relationship ceases.

I acknowledge that confidential information includes all information relating to the Client, their premises and  personal property.

I understand that I am bound by a duty of confidential and agree to adhere to this.

Code of Conduct and the requirement of the Data Protection Act 1998.

Cleaner print name:

Signature:

Date:

Health and Safety Arrangements and Procedures

- Supervision

- Risk Assessments

- Workplace & Site Safety, and Welfare

- Control of Substances Hazardous to Health

- Asbestos

- Manual Handling

- First Aid

- Accident Reporting

- Electricity

- Noise

- Vibration

- Working at Height

- General Work Equipment

- Fire

- Computer Screens

- Personal Protective Equipment

Arrangements and Procedures

Training and Supervision

It will be the policy of Eco-Clean and Maintenance Ltd (“the Business”) that health and safety information relevant to the activities at hand is given to all employees and sub-contractors.

Ongoing training will be provided to ensure that employees at all levels are:

- competent to carry out their duties, to operate specialist tools, plant and work equipment; and

- aware of their health and safety responsibilities.

Decisions relating to ongoing training of employees will be reviewed on a regular basis. The person with day-to-day responsibility for Health and Safety will be responsible for identifying and implementing health and safety training needs.  Records of the training will be kept on each employee’s individual file.

Checks will be made to ensure that sub-contractors are competent to carry out the tasks allocated to them and that they have health and safety management systems in place appropriate to their work.

Risk Assessments

(Reg. 3 Management of Health and Safety at Work Regulations, 1999)

The Management Team will ensure that all hazardous work activities undergo a suitable and sufficient Risk Assessment.  Upon the establishment of the level of risk, preventative measures will be introduced, maintained and revised as needed.

Workplace Safety and Welfare

(The Workplace (Health, Safety and Welfare) Regulations 1992; Construction (Design & Management) Regulations 2015)

The management will ensure that all workplaces meets the health, safety and welfare needs of all those who will use them, including contractors, and wherever appropriate, people with disabilities. Where works are to take place where members of the public have access, measures will be taken to ensure that they are not adversely affected.

Control of Substances Hazardous to Health

(COSHH 2002)

Where needed, Risk Assessments and all Material Safety Data Sheets of substances used will be kept at the Business’ office.  From the Risk Assessments the Management will instigate the principles of good practice for the control of exposure as detailed in Schedule 2A Regulation 7(7).   No employee will introduce any substance without the specific consent of their Manager.

Asbestos

- (Control of Asbestos at Work Regulations 2012)

In the event that any substance suspected to be, or containing, asbestos is found during the course of the Business’ works, all works in the area will cease immediately to avoid any exposure.  Work will be suspended in that area until the substance has been identified and if appropriate made safe/removed by specialist contractors. No works will be carried out that may disturb suspect substances without a suitable Asbestos Survey having been done by a specialist contractor.

Manual Handling

- (Manual Handling Operations Regulations, 1992)

The designated H&S person will assess all manual handling operations within the workplace, and where possible change the nature of any task or provide mechanical aids in order to reduce or lighten the manual handling of loads.  They will ensure that adequate Risk Assessments are carried out to identify hazards associated with manual handling and ensure that where needed, lifting aids are provided.

First Aid

(Health and Safety (First Aid) Regulations, 1981)

The designated H&S person shall ensure that all employees have access to adequate First Aid provision at all times. The level of cover shall be determined by risk assessment.

Accident Reporting

(Reporting of Injuries, Diseases, Dangerous Occurrences Regulations 2013)

All accidents and incidents will be recorded in an accident book and personal details kept secure to comply with Data Protection legislation. Any reportable accidents, incidents, or dangerous occurrences will be reported to the HSE.

Electricity

(Electricity at Work Regulations, 1989)

It is the policy of the Business that wherever possible, all hand tools will be battery powered or 110v.  Where this is not possible a Residual Current Device (RCD) will be used.

Noise

(Control of Noise at Work Regulations, 2006)

The person in charge of a working area/site will ensure that noisy works do not cause a nuisance to others in the vicinity of their works area. They will always ensure that suitable ear protection is freely available to anyone who needs it.

Vibration

(Control of Vibration at Work Regulations 2005)

The designated H&S person will ensure that where the use of vibration causing hand tools cannot be reduced, suitable safeguarding procedures are brought in, including where appropriate, anti-vibration gloves.

Working at Heights

(Work at Heights Regulations 2005)

Where the business activities involve ‘work at height’ the designated H&S person will assess the task beforehand and will consider the most suitable means of access.    Suitable control measures such as exclusion zones will be put in place to ensure the safety of any others who will be in the vicinity.

General Work Equipment

(The Provision and Use of Work Equipment Regulations (PUWER) 1998) (Lifting Operations and Lifting Equipment Regulations (LOLER) 1998)

It will be the designated H&S person’s responsibility to ensure:

- Suitable equipment is supplied for the tasks to be done (Reg. 4)

- That the equipment will be maintained in an efficient state, in efficient working order and in good repair (Reg. 5)

- That all equipment and plant shall be inspected as required by Regulation 6

- That those tasked with using the equipment are competent to use it. (Regs. 8 & 9)

Fire

(The Regulatory Reform (Fire Safety) Order 2005)

It will be the responsibility of the designated H&S person to ensure adequate fire extinguishers are available at all work locations. All fire extinguishers are to be checked annually by a specialist contractor.

Where any hot works are to take place, they will ensure a suitable fire extinguisher is to hand at all times.

Computer Screens

(The Health and Safety (Display Screen Equipment) Regulations, 1992)

The designated H&S person will ensure suitable assessments are carried out for all persons who use display screen equipment. The assessments will consider the amount of time a person uses a VDU and the work done, the usability of their workstation and general working environment.

Personal Protective Equipment (PPE)

(The Personal Protective Equipment at Work Regulations (PPE) 1992)

The designated H&S person will ensure that all individuals have access to a sufficient supply of PPE when required.

These arrangements and procedures will be reviewed annually or when there is a change in circumstances, in work practices or the introduction of new legislation.

| Name: | Angela Varney |
| --- | --- |
| Position: | Director |
| Signature: |  |

Fire Safety Policy 21/22

Introduction

Eco-Clean and Maintenance (“the Company”) is committed to providing a safe working environment for its staff and visitors.  For this reason, the Company has formulated this policy to facilitate compliance with the Company’s legal obligations under The Regulatory Reform (Fire Safety) Order 2005 (“Fire Safety Order”).

Policy objectives

To provide a safe and healthy working environment for all staff and visitors.

To minimise the risks to Company premises from fire.

To manage fire risks in accordance with the requirements of the Fire Safety Order.

To comply with the requirements of the Health and Safety at Work Act etc1974, the Management of Health and Safety at Work Regulations 1999 and the Fire Safety Order.

To address obligations under the Fire Safety Order that require the Company to:

Develop a policy to minimise the risks associated with fire.

Reduce the risk of an outbreak and subsequent spread of fire.

Provide means of escape.

Demonstrate preventative action.

Maintain documentation and records in respect of fire safety management.

The Responsible Persons

The Company has appointed Angela Varney as the ‘responsible person’.  The responsible person’s duties are to ensure the safety of staff and visitors by:

Carrying out (or ensuring that a competent person carries out) a Fire Risk Assessment. The Fire Safety Risk Assessments will take into consideration everyone who may come onto the premises, whether they are employees or visitors, and particular attention will be paid to people who may have a disability or anyone with special needs.  Making sure, as far as is reasonably practical, that everyone on the premises, or nearby, can escape safely if there is a fire.

Preparing a written Emergency Action/Evacuation Plan for the building to be displayed at various locations about the premises.

Preparing Personal Evacuation Plans for disabled persons (if relevant).

Fire Marshals

The Company will appoint competent persons to act as Fire Marshals. The Fire Marshals’ duties will include:

Making contact with the emergency services.

Assisting in evacuations/fire drills.

Carrying out regular checks on all fire safety equipment including emergency lights and alarms.

Ensuring that emergency escape routes are kept clear at all times and that doors designated as Fire Escapes are operable.

The names and duties of all competent persons will be displayed on the safety notice board.

Communication

The Company will ensure that all persons employed either as direct employees or contractors are provided with all relevant information related to fire safety.  The Management of the Company will consult with the employees (where relevant) on all relevant matters of fire safety policy and arrangements and will ensure staff are kept informed of any changes that are made to fire safety procedures.

Training

Upon commencement of employment all employees (where relevant) will be given training on fire safety and will receive refresher training as appropriate.

All employees will be instructed to report any defective or missing equipment to their line manager.

All employees will receive instruction on their role in the case of an emergency.

[It shall be Company policy that all staff will be trained in the use of fire extinguishers whether or not they have been given specific firefighting duties.]

Further training may be required if there are any changes that may affect fire safety. All training will be provided during normal working hours.

Equipment/Testing

The fire evacuation procedures will be practiced every six months.

Firefighting equipment will be provided. In general, this means fire extinguishers, but additional provision of fire blankets, hoses or sprinklers may be made where deemed appropriate by the findings of the risk assessment.

All fire safety equipment will be serviced by a competent person and the service periods will be scheduled in accordance with the manufacturers’ instructions.

An appropriate fire detection and alarm system will be provided. The type and extent of the alarm system provided will be based on the findings of the risk assessment.  Alarm systems will be tested regularly. Staff will be told when tests are scheduled.

Emergency lighting will be provided for escape routes where applicable. The location and type will be determined by the findings of the risk assessment.

Operation of fire exit doors, including any automatic closers, will be tested and recorded in the fire log on a weekly basis.

Any other safety systems provided, such as emergency lighting and fire doors, will be checked regularly to ensure correct operation

Procedures

The Company has introduced the following procedures in order to maintain high standards of fire safety:

Emergency escape routes will be established and kept free from obstruction at all times;

Fire exit doors will be kept in good working order and unlocked at times the premises are occupied;

The risk of fire spreading through the building will be controlled by the provision of fire/smoke resisting doors;

Signs and notices will be displayed, giving appropriate instructions to employees and others in the event of a fire;

Signs will be provided to indicate the position of fire extinguishers, fire alarm call points and to indicate the emergency exit routes.

Records

The Company will record its staff training including fire drills, and the findings of its periodic tests and checks.  Such records will include all attendees, fire drill evacuation times and any comments.  The Company will keep the following records:

Records of weekly tests of fire alarms, fire exits.

Records of weekly flow tests of sprinkler systems (where fitted).

Records of wet and dry tests of dry rising mains (where fitted).

Record of annual inspection and test of all firefighting equipment.

Records of periodic tests of emergency lighting (where fitted).

Records of all scheduled and unscheduled maintenance of fire detection and alarm systems.

Records of the inspection, risk assessment and maintenance of workplace and electrical equipment, of storage of hazardous substances and of any other hazards identified with fire safety (where appropriate).

This Policy will be reviewed annually and updated as necessary. The management team endorses this policy and is fully committed to its implementation.

| Name: | Angela Varney |
| --- | --- |
| Position: | Director |
| Signature: |  |
| Date: | September 2023 |
| Date for Review: | September 2024 |

| Issue No | 005 |
| --- | --- |
| Amendments from previous | N/A |
| Other comments | N/A |

Health and Safety Policy 2023/2024

Introduction

Eco-Clean and Maintenance Ltd (“the Company”) recognises and accepts its responsibilities under the Health and Safety at Work etc Act 1974 including the responsibility to:

provide and maintain a safe and healthy place of work

provide adequate information, instruction, training and supervision

provide and maintain plant and equipment and safe systems of work

ensure safe access to and from the places of work

work to prevent accidents and work-related ill health

General Health and Safety

The overall responsibility for health and safety lies with the Managing Director, Angela Varney, has day to day responsibility for managing health and safety.

The Management are committed to achieving the highest standards of health and safety throughout the Company.

The Management are also committed to complying with the requirements of the Management of Health and Safety at Work Regulations 1999 and other Regulations that apply to the Company’s work activities.

The Management team will ensure that assessments of all areas of work activities are carried out regularly, in order to identify hazards and work to prevent instances of injury, disease and dangerous occurrences arising.

The Management are also committed to ensuring that the work done by the Company does not adversely affect the health and safety of any contractors or of members of the public.

The Management are fully committed to providing safe and healthy working conditions and adequate welfare facilities for all employees.

The Company will strive to maintain excellence in health and safety matters and in this respect, employees and others are encouraged to co-operate with the management in all safety matters, to identify hazards and reduce the risk which may exist during work activities and to report any condition which may appear dangerous or unsatisfactory.  The Company will at all times consult with the employees on these matters.

The Senior Management will, so far as reasonably practicable, ensure that the Company provides adequate financial resources to meet these objectives.

Copies of this policy are to be available to all Company employees and other interested parties.

Directors Duties:

The Director will have at least basic knowledge and understanding of the Health and Safety at Work etc Act 1974 and its associated Regulations and Approved Codes of Practice.

It will be the responsibility of all the Director to keep all employees advised as to their responsibilities in respect of health and safety matters.

In order to protect the safety and health of employees and others affected by the Company's operations, the Director will:

Take reasonable steps to familiarise themselves with the hazards and risks associated with working at the Company and with the precautions which need to be taken to eliminate or control those risks.

Establish procedures to deal with any emergencies.

Appoint a suitably trained and competent person to assist them in carrying out their health and safety duties.

Ensure that employees receive sufficient training and information so that they can carry out their duties safely and competently. Ensure adequate funds and facilities are available for this purpose. Before entrusting work tasks to employees, take into account their capabilities as regards health and safety and ensure that suitable Risk Assessments are carried out on any hazardous activity.

Initiate the timing and annual review of the Health and Safety Policy and ensure it is promoted to all employees and others working on behalf of the Company.

Ensure that all employees carry out the health and safety responsibilities allocated to them.

Ensure the safety performance of the Company is monitored and take action to remedy any identified deficiencies.

Ensure that adequate provision is made for welfare facilities and that adequate first aid provisions are made.

Ensure that all necessary PPE is provided to employees, and that instruction is given on its use.

Designated Health & Safety Person’s Duties:

To ensure that all the Company Directors, Supervisors and Staff are aware of their individual Health and Safety responsibilities.

To initiate and/or recommend any changes, developments and amendments to the policy as and when necessary.

To inform the Health and Safety Executive of all notifiable accidents.  Investigate any accidents or dangerous occurrences and recommend means of preventing re-occurrence.

To arrange appropriate training for all employees.

To create and maintain a Training Matrix for all staff.

To ensure that Risk Assessments (including where appropriate, COSHH, Noise, Manual Handling etc) are carried out as needed.

To ensure follow up action as needed.

To promote an interest and responsible attitude towards Health and Safety matters throughout the Company.

This policy will be reviewed annually or when there is a change in circumstances, in work practices or the introduction of new legislation.

| Name: | Angela Varney |
| --- | --- |
| Position: | Director |
| Date: | September 2023 |
| Signature: |  |

This is the Health and safety policy statement of

ECO Clean and maintenance Ltd

2023/2024

It is the policy of Eco-Clean and maintenance Ltd (“the Company”) to foster a positive health and safety culture throughout the Company because we believe that high standards of health and safety are a moral and commercial pre-requisite.

- The Company is committed to:

- providing adequate control of the health and safety risks arising from our work activities

- working to prevent accidents and work-related ill health

- providing and maintaining safe plant and equipment

- maintaining safe and healthy working conditions, and adequate welfare facilities

- ensuring safe handling and use of substances

- using and maintaining the proper safe plant and equipment needed for each task, including all Personal Protection Equipment where needed

- ensuring all employees are competent to do their tasks, and to give them adequate training

- ensuring the safety of our clients/customers at all times

- reviewing and revising this policy statement annually

Our stated aims and objectives for the year 2023 are:

- To ensure all H&S documentation is up to date

- To update all Risk Assessments

- To continue to work to ensure compliance

- Implementation, maintenance and review

The Director, Angela Varney, accepts overall responsibility for all Health and Safety within the Company and is responsible for all policy implementation.

Review date:	September 2024

Safeguarding Policy

- Introduction and Purpose

From time to time Eco-Clean & Maintenance Ltd works with clients who operate within a setting that brings our employees into contact with children, young people or vulnerable adults. These include schools, colleges, universities and attractions.

The Company is committed to working with these clients to ensure our employees safeguard and promote the welfare of children or young and vulnerable adults in the workplace.

Everyone working within these settings has a responsibility for safeguarding children and young or vulnerable adults. Our staff members will be made aware that it is important to maintain an attitude of ‘It could happen here’ where safeguarding is concerned. When concerned about the welfare of a child, staff members should always act in the interests of the child.

The purpose of this policy is to make sure that the actions of any adult in our employ, whilst performing the works for which they are employed by us, are transparent and safeguard the children, young people and vulnerable adults that they may encounter.

We will be guided by the client and the Designated Safeguarding Lead (DSL) within the client’s workplace.

- Good practice guidelines and staff code of conduct

To meet and maintain our responsibilities towards safeguarding we expect our employees to:

- treat everyone with respect;

- set a good example by conducting themselves appropriately;

- ensure that, whenever possible, there is more than one adult present where they are working in the vicinity of vulnerable adults or children;

- be aware that any physical contact with vulnerable adults or children may be misinterpreted, no matter how well intentioned;

- operate within Eco-Clean’s principles and guidance and any other specific procedures;

- challenge unacceptable behavior and report all allegations or suspicions of abuse to the appropriate person at the client premises;

- share any concerns with your supervisor/area manager and/or the DSL at the client premises without delay.

Employees must not:

- have inappropriate physical or verbal contact with children, young people or vulnerable adults;

- allow yourself to be drawn into inappropriate attention seeking behavior or make suggestive or derogatory remarks or gestures in front of vulnerable adults, children or young people;

- jump to conclusions about others without checking facts by discussing concerns with your immediate line manager;

- either exaggerate or trivialize vulnerable adults or child or young people abuse issues;

- show favoritism to any individual;

- What is abuse?

It is generally accepted that there are four main forms of abuse. The following definitions are taken from “Working Together to Safeguard Children” a Department for Education publication.

- Physical Abuse

This includes anything from a hand slap to death by suffocation. Injuries may be caused by blows, punches, kicks, shakes, bites, belts, scalds, burns, suffocation, drowning or poisoning. Physical harm may also be caused when a parent or carer feigns the symptoms or deliberately causes ill health to a vulnerable adult or child they are looking after, Physical abuse, as well as being the result of a deliberate act, can also be caused through the omission or the failure to act and protect.

- Neglect

Neglect is the persistent failure to meet a child’s basic physical and/or psychological needs, which is likely to result in serious impairment to their health or development. It may involve a carer failing to protect a child from exposure to any kind of danger, including cold or starvation, or extreme failure to carry out important aspects of care, resulting in significant impairment of the child’s health or development.

- Sexual Abuse

Forcing or enticing a child or young person to take part in sexual activities, whether or not they are aware of what’s happening. This includes rape, incest and all forms of sexual activity involving children, including encouraging children to look at, or to be involved in the production of pornography material, prostitution or encouraging children to behave in sexually inappropriate ways.

- Emotional Abuse

This is persistent emotional ill treatment which is likely to cause serious harm to the child’s emotional development. This may involve conveying to children that they are worthless, unloved, inadequate and cause children to feel frightened, in danger, be exploited or corrupted.

- What to do if someone discloses an abusive act or experience to a Eco-Clean & Maintenance Ltd employee

If a vulnerable adult, child or young person confides to a Eco-Clean & Maintenance Ltd employee that they are being or have been abused then they have placed that person in a position of trust. The employee concerned must:

- be clear that they cannot keep secrets or promise confidentiality and that they must pass on information if they think a child or young person has been or is being harmed, or is at risk of being harmed, in some way;

- react calmly. Do not panic as this may frighten or silence the person;

- make a full record of what is being said, heard or seen as soon as possible and report it to the Clients DSL and their respective supervisor/area manager.

- DO NOT DELAY IN PASSING INFORMATION ON.

It is not the sole responsibility of Eco-Clean & Maintenance Ltd to decide whether abuse has taken place. This will be decided after a full investigation and the relevant Clients’ DSL has been informed. It is the responsibility of our employees to act if there is cause for concern, in order that the client and appropriate agencies can investigate and take the necessary action to protect a vulnerable adult or child.

All concerns should be reported the same working day.

- Responding to allegations of abuse or inappropriate or dangerous behavior against a member of Eco-Clean Cleaning staff

Regardless of the age of the individual person, if an allegation of abuse or inappropriate conduct is made against an Eco-Clean employee then it must be reported immediately to the client’s DSL and their respective supervisor/area manager.

Concerns about employees must be treated with the same rigor as other concerns. If there are concerns that child abuse has taken place then this information will be passed to social services and/or the police for a full investigation by the client, supported by the relevant Eco-Clean & Maintenance Ltd Senior Manager.

The welfare of the child, young person or vulnerable adult is paramount. Even when the presenting circumstances appear not very serious, they must be followed up, and they are examined objectively by someone independent of Eco-Clean & Maintenance Ltd.

Eco-Clean & Maintenance Ltd.’s Senior Managers will also need to refer to any disciplinary policy and procedure and decide whether the member of staff should be suspended pending a full investigation.

- After an allegation

After an allegation or suspicion or concern has been investigated, there are likely to be strong feelings among the staff, service users, parents and maybe the wider community, which will need to be addressed. There are likely to be issues of rumor or fact, guilt and blame – if suspicions have been around for some time, impact on individuals, of the nature of what occurred and to whom. Eco-Clean Senior Management team will support the client and employee in considering the best way of managing this.

- Training

Our employees will undertake basic safeguarding training as required by our client organization.

- Recruitment

Enhanced checks with the Disclosure and Barring Service (DBS) will be conducted on Eco-Clean staff on appointment, when they work within client settings that operate with children, young people and vulnerable adults. We will be guided by our client requirements with regards to the frequency of DBS checks.

- Review

This policy will be reviewed regularly and in line with our client’s requirements.

- Authorization

The authority for this policy is vested in the Managing Director.

Angela Varney

Managing Director

Document Control

A hard copy of this policy is kept at Eco-Clean & Maintenance Ltd Head Office.

Unit 2, Whitehouse Farm, Oxford Road

Princethorpe, Rugby CV32 9QD

Environmental Policy - Eco-Clean & Maintenance Ltd

Eco-Clean believe that businesses are responsible for achieving good environmental practice and operating in a sustainable manner.

We are therefore committed to reducing our environmental impact and continually improving our environmental performance as an integral and fundamental part of our business strategy and operating methods.

It is our priority to encourage our customers, suppliers and all business associates to do the same.  Not only is this sound commercial sense for all; it is also a matter of delivering on our duty of care towards future generations.

Our policy is to

- Wholly support and comply with or exceed the requirements of current environmental legislation and codes of practice.

- Minimise our waste and then reuse or recycle as much of it as possible.

- Minimise energy and water usage in our buildings, vehicles and processes in order to conserve supplies, and minimise our consumption of natural resources, especially where they are non-renewable.

- Operate and maintain company vehicles (where appropriate) with due regard to environmental issues as far as reasonably practical and encourage the use of alternative means of transport and car sharing as appropriate.

- Apply the principles of continuous improvement in respect of air, water, noise and light pollution from our premises and reduce any impacts from our operations on the environment and local community.

- As far as possible purchase products and services that do the least damage to the environment and encourage others to do the same.

- Assess the environmental impact of any new processes or products we intend to introduce in advance.

For any issues related to Environmental Policy linked to this business please contact:

…………………………………………………
Angela Varney

Managing Director

01926 915390

Eco Clean Ethics Policy

  - Overview

Eco-Clean purpose for this ethics policy is to establish a culture of openness, trust and integrity in business practices.  Effective ethics is a team effort involving the participation and support of every Eco-Clean employee.  All employees should familiarize themselves with the ethics guidelines that follow this introduction.

Eco-Clean is committed to protecting employees, partners, vendors and the company from illegal or damaging actions by individuals, either knowingly or unknowingly.  When Eco-Clean addresses issues proactively and uses correct judgment, it will help set us apart from competitors.

Eco-Clean will not tolerate any wrongdoing or impropriety at any time.  Eco-Clean will take the appropriate measures act quickly in correcting the issue if the ethical code is broken.  Any infractions of this code of ethics will not be tolerated.

  - Purpose

Our purpose for authoring a publication on ethics is to emphasize the employee’s and consumer’s expectation to be treated to fair business practices.  This policy will serve to guide business behavior to ensure ethical conduct.

  - Scope

This policy applies to employees, contractors, consultants, temporaries, and other workers at Eco -Clean including all personnel affiliated with third parties.

  - Policy

  - Executive Commitment to Ethics

    - Managers within Eco-Clean must set a prime example.  In any business practice, honesty and integrity must be top priority for executives.

    - Managers must have an open door policy and welcome suggestions and concerns from employees.  This will allow employees to feel comfortable discussing any issues and will alert executives to concerns within the work force.

    - Managers must disclose any conflict of interests regard their position within Eco-Clean.

  - Employee Commitment to Ethics

    - Eco-Clean employees will treat everyone fairly, have mutual respect, promote a team environment and avoid the intent and appearance of unethical or compromising practices.

    - Every employee needs to apply effort and intelligence in maintaining ethics value.

    - Employees must disclose any conflict of interests regard their position within Eco-Clean.

    - Employees will help Eco-Clean to increase customer and vendor satisfaction by providing quality product s and timely response to inquiries.

  - Company Awareness

    - Promotion of ethical conduct within interpersonal communications of employees will be rewarded.

    - Eco-Clean will promote a trustworthy and honest atmosphere to reinforce the vision of ethics within the company.

  - Maintaining Ethical Practices

    - Eco-Clean will reinforce the importance of the integrity message and the tone will start at the top.  Every employee, manager, director needs consistently maintain an ethical stance and support ethical behavior.

    - Employees at Eco-Clean should encourage open dialogue, get honest feedback and treat everyone fairly, with honesty and objectivity.

    - Eco-Clean has established a best practice disclosure committee to make sure the ethical code is delivered to all employees and that concerns regarding the code can be addressed.

  - Unethical Behavior

    - Eco-Clean will avoid the intent and appearance of unethical or compromising practice in relationships, actions and communications.

    - Eco-Clean will not tolerate harassment or discrimination.

    - Unauthorized use of company trade secrets & marketing, operational, personnel, financial, source code, & technical information integral to the success of our company will not be tolerated.

    - Eco-Clean will not permit impropriety at any time and we will act ethically and responsibly in accordance with laws.

    - Eco-Clean employees will not use corporate assets or business relationships for personal use or gain.

- Enforcement

  - Any infractions of this code of ethics will not be tolerated and Eco-Clean will act quickly in correcting the issue if the ethical code is broken.

  - Any employee found to have violated this policy may be subject to disciplinary action, up to and including termination of employment

Method Statements

Safe Systems of Work

Method Statements

Method Statement

Task

General cleaning / damp dusting of horizontal and vertical surfaces

Equipment

Blue coloured cloth

Blue coloured bucket

Gloves

General purpose detergent or general surface cleaner

Method

- Wash hands and put on gloves

- Prepare cleaning solution in well ventilated area as per manufacturer’s instructions

- Always add product to water

- Remove items from surface to be cleaned

- To damp dust flat surfaces wipe in straight lines with the edges first, then fill in using a figure eight (8) pattern to complete the area

NOTE Frequently turn the cloth rinsing in the cleaning solution and change the solution soon as it becomes soiled

- Use chosen cleaning solution to remove stubborn stains / grease marks

- Replace items moved back onto clean surface.  After use, all equipment should be checked, rinsed and returned to cleaning cupboard

- Remove gloves and wash hands

Method Statement

Task

High dusting

Equipment

Long handled flick duster / Dome Brush Duster

Method

- Wash hands and put on gloves

- Using extending flick duster / Dome Brush Duster draw the head along top Surfaces, bookshelves, ledges, pipes, light fittings

NOTE Do not stand on chairs or use ladders (refer to risk assessment)

High dusting should be completed first allowing any dust to be cleared up in next cleaning procedure

- Clean flick duster by placing the head into a bag, sealing the bag around the head and shaking to remove dust particles.  Remove from bag and store in cleaning cupboard.  Wash out duster on a regular basis to prolong the life of the equipment and make cleaning more efficient.

- Remove gloves and wash hands

Method Statement

Task

Damp mopping of floors

Equipment

Colour coded bucket – Blue for general floors Red for toilets

Colour coded mop head / handle: -

Blue for general floors, Red for Toilets

Gloves / Warning signs

Appropriate floor cleaning detergent

Method

- If mopping stairs make sure the area is cordoned off and warning signs placed in the area you are working in

- Wash hands and put on gloves

- Display warning signs

- Dust control the floor area

- Prepare the cleaning solution in well ventilated area, always put water in first then add cleaning product

- Submerge mop into the cleaning solution and remove excess water from the mop in the wringer

- Mop the floor in 1 – 2 metre square sections

- Mop edges of the floor with a straight stroke first, complete the area using a figure eight (8) motion, rinse the mop frequently leaving the area as dry as possible

- Avoid splashes to other surfaces and remove any splashes as they occur

- On completion, remove all equipment and store in cleaning cup board

NOTE leave sign until floor is dry, remove gloves and wash hands

Method Statement

Task

Cleaning all Sanitary appliances (toilets)

Equipment

Red coloured cloths for toilets, Blue for general

Red Gloves

Colour coded spray bottle

Toilet brush

Warning sign

General surface cleaner / toilet bowl cleaner

Supply of toilet paper / hand towels or applicable

Method

- Wash hands and put on gloves

- Assemble cleaning equipment

- Place warning signs

- Ventilate area (for example open window or door)

- Dry sweep floor and empty bins of wastepaper

- Flush the toilet with the seat down, then lower the water level by pushing the water down the u-bend using the toilet brush exposing the water line

- Apply the cleaning solution to the toilet bowl including under the rim.  Leave to soak while you continue to clean the rest of the area

- Using the Blue coloured cloth apply cleaning solution to the sink area.  Clean sink starting at the taps, inside the sink and then the outside bowl area.  When completely clean, rinse and dry area thoroughly with a dry Blue cloth

- Wipe clean any areas where splashes have occurred i.e. tiles, shelves, sills and pipe work

- Return to the toilet bowl and using the brush rub all surfaces from the rim to the water line then into the u-bend.  Continue doing this process until all cleaning product has been removed, flushing the toilet to remove any excess.  Using a Red coloured cloth completely wipe the toilet, starting at seat hinge working across the seat through to the outside of the toilet bowl.

- Using the Blue cloth wipe over any fixings – toilet roll holder etc – dry and buff with a dry red cloth and then replenish supplies

- Damp mop floor using method for damp mopping hard surfaces

NOTE Leave warning signs until area completely dry

- Remove gloves, wash hands and return all equipment to the cleaning cupboard making sure that all equipment is washed, wrung out and put to dry.  All Red and Blue cloths are put to launder and replaced with clean ones.

Mop to be stood with head upward.  Do not leave mop head standing in bucket

Method Statement

Task

Kitchen cleaning

Equipment

Green coloured cloth

Green coloured bucket

Green colour mop / handle

Gloves / warning sign

Abrasive pad

General purpose cleaner

Method

- Wash hands, put on gloves and place warning signs

- Make sure surfaces are clear by removing any crockery to sink or dishwasher

- Wash crockery using manual cleaning methods if applicable to contract

- Using Green coloured cloth wipe all surfaces, removing any items on surface and wiping underneath, frequently rinsing cloth as per task.  When wiping stainless steel, buff to a shine after cleaning using kitchen paper or a dry Green colour cloth

- Wipe over any cupboards, including tops and side of doors, bins and tiles

- Rinse sinks and dry thoroughly using a Green coloured cloth

- Sweep or vacuum the floor

- Mop as per method for hard floor cleaning

- Remove all equipment to cleaning cupboard making sure it is rinsed and dry.  Put all Green coloured cloths to launder and replace with dry ones

Method Statement

Task

Vacuum carpeted areas

Equipment

Vacuum

Warning signs

Extension lead

Method

- If working on stairs, close area and place warning signs

- Place warning sign in any area you are working in

- Check equipment visually, check plug, socket, wire and casing of electrical appliance

- Remove any furniture that is necessary and pull-out chairs

- Plug in appliance, leaving cable to edge of room and behind you, switch on and work in areas 1-2 metre squared areas

- Attach nozzle for cleaning along skirting and around furniture that cannot be reached

- Carry on this process until area is completed

- Replace furniture, aligning chairs to desks or tables

- Switch off appliance, turn off switch at socket and remove plug

- Wind flex around the appliance to prevent any kinks or knots in wire

- Check if bag is full and replace if necessary

- Remove appliance to cleaning cupboard

- When vacuuming stairs work from the bottom to the top making sure you have enough pipes to reach each stair section.  Use extension pipe if needed

- NOTE Never balance vacuum on stair treads

Method Statement

Task

Dry sweeping hard floors

Equipment

Flat sweeper or broom

Dustpan / brush

Gloves

Warning signs

Colour coded spray bottle

Face mask

Black refuse sack

Method

- Wash hands and put on gloves and face mask

- Place warning signs

- Remove any large debris to black refuse sack

- Start at edges of the room sweep in straight strokes towards the entrance of the room.  Continue through the centre area keeping all debris in one place.  Remove debris with dustpan and brush to black refuse sack

NOTE If area is dusty spray with water to prevent further dust

- Remove rubbish to refuse area

- Return equipment to cleaning cupboard making sure equipment is clean

Method Statement

Task

Refuse removal

Equipment

Black refuse bags

White bin liners

Gloves

Colour coded cloth and buckets (see relevant method statement)

Method

- Check contract specification for re-cycling procedures

- Wash hands and put on gloves

- Check for any spills

- If spills have occurred wipe out bin with wet cloth and replace liner

- If recycling is in place and cleaning spec states empty wastepaper / card to a separate bag for recycling

- Continue until all bins have been checked

- Remove waste to specified area

- Keeping recycling separate, empty general waste (including bags) into specified bin

- Recycling waste emptied into specified receptacle (without black bag)

- Remove glove and wash hands

- Return any equipment to the cleaning cupboard

Method Statement

Task

Wall washing

Equipment

Blue coloured cloth

Blue coloured bucket

Blue coloured mop / handle

Gloves

Safety goggles

Labelled spray bottle

Abrasive pads

Access equipment – extending pole with attachments

Masking tape to cover electrical sockets

Warning signs

Method

- Wash hands and put on gloves

- Put on goggles

- Display warning signs

- Assemble equipment, and check for safety

- Cover electrical sockets with masking tape

- Ventilate the area

- Prepare cleaning solution – refer to manufacturer’s instructions

- High dust to remove loose debris and dirt (refer to high dusting method)

- Apply the cleaning solution to the wall wiping from bottom to top.  Allow a short time for the cleaning agent to work then rinse.  Wipe wall from top to bottom leaving the wall as dry as possible.

- Remove drips and runs as they occur

- When task is complete remove tape from sockets

- All equipment should be cleaned and checked after use

- Remove gloves and wash hands

Method Statement

Task

Spot cleaning

Method

- Wash hands and put on gloves

- Remove furniture or pull-out racking

- Rinse and wring out cloth in cleaning solution

- Wipe the soiled area from the outside of the mark to the centre

- Rinse with clean water

- Remove drips and runs as they occur

- Dispose of the cloth when the task is completed

- When area is dry replace furniture or racking

- Wash, check and put away cleaning equipment

- Remove gloves and wash hands

Method Statement

Task

Oven cleaning

Equipment

Green coloured cloth

Green coloured bucket

Gloves

Safety goggles

Scraper

Colour coded labelled spray bottle

Non-abrasive pads

Warning signs

Face mask

Kitchen degreaser

Oven cleaner

Method

- Wash hands and put on PPE – gloves, mask, goggles

- Place warning signs

- Ensure oven is at the correct temperature for the job

- Remove all detachable parts to a safe cleaning area

- Remove internal shelves and panels

- Ventilate the area

- Remove any loose soil / dirt

- Prepare cleaning solution – refer to manufacturer’s recommendations

- Dampen or rinse Green coloured cloth in cleaning solution and wring out well

- Thoroughly wipe clean all detachable parts, rinse and dry

- For stubborn stains / soil use oven cleaner / degreaser as per manufacturer’s instructions

- Allow sufficient contact time for solution to soften soil

- Remove all cleaning solution, rinse and dry

- Clean inside and outside the oven and its immediate environment

- Re-assemble the detachable parts correctly

- Re-connect mains supply and re-light pilot if necessary

- Clean and check all equipment

- Dispose of used cloths

- Remove gloves and wash hands

Cleaning with Colour Coding

Colour coding is the system we use to enable us to limit cross contamination to a minimum, thus reducing the risk to ourselves and our service users from bacteria related infections.

RED CLOTHS AND EQUIPMENT

These are for use in sanitary appliances (toilets)

GREEN CLOTHS

These are to be used only for cleaning kitchens

BLUE CLOTHS AND EQUIPMENT

These are to be used in general areas – corridors, office, stairways etc

Method Statement

Task

Lift cleaning

Equipment

Blue coloured cloth

Blue coloured bucket

Gloves

General purpose detergent or general surface cleaner

Vacuum cleaner

Isolation key

Method

- Open the door

- Isolate all controls for the lift

- Wipe all internal walls and rails using the appropriate product

- Wipe mirror

- Vacuum floor – standing at door and working inwards.  NEVER take vacuum into the lift

- When finished turn isolator key off

- Clean exterior of lift door

- All equipment should be cleaned and checked after use

NEVER CLEAN LIFT UNLESS ALL CONTROLS HAVE BEEN ISOLATED

Method Statement

Task

Using Step Ladder

Equipment

Step ladder

Pre-User check

- Check the stiles – make sure they are not bent or damaged

- Check the feet.  Ensure the feet are also checked when moving from soft/dirty ground to a smooth, solid surface

- Check the rungs

- Check any locking mechanisms

- Check the stepladder platform

- Check the steps or treads

Method

- Check all four feet are in contact with the ground

- Do not over-reach

- Do not stand on the top step

- Position the ladder to face the work activity and not side on.

- Maintain three point of contact at the working position.  This means two feet and one hand, or when both hands need to be free for a brief period, two feet the body supported by the stepladder.

Use ladder on level ground, on clean solid surfaces, where they will not be struck by vehicles, where they will not be pushed over by hazards such as windows or doors, where it is a risk to the general public and where it has been secured.

Risks Assessments

COMPANY:                                                                                                                                            DATE OF RISK ASSESSMENT: Sep 2023

BY: CONTRACT MANAGER

Date of REVIEW: Jan 2023

Signed Management: D.Gibson

| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| --- | --- | --- | --- | --- | --- | --- |
| Slips, trips and falls | Staff and others, risk injuries such as fractures and bruising if they trip over objects, trailing leads or slip-on spillages or wet floors. | Good housekeeping at client’s premises. Warning signs placed in wet floor areas and when vacuuming. Client to instruct staff to keep off wet floors. Cleaners to use electrical socket nearest to where working. Close area whilst working until floor dry. Instructed to wear sensible shoes. EG flat shoes with good grip. | To further reduce the risk of slips | Contract Manager / Company Director | On Going | On-going training of staff |
| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| Contact with chemicals | Staff risk skin problems such as dermatitis and eye damage from direct contact with chemicals. Vapour could cause breathing problems | Staff are asked when they begin employment if they have any medical conditions / allergies relating to chemicals. Long handled mops and brushes, face masks, goggles and rubber gloves are provided, and staff trained in their use. All staff are trained in the safe use and storage of cleaning chemicals Introduction of micro fibre cleaning. | 1. Investigate changing chemicals marked “irritant” with milder alternatives. 2. Staff reminded to report any health problems they think may come from cleaning chemicals and to check for dry, red or itchy hands. 3. Staff are reminded to wash gloves after use | Contract Manager / Company Director All staff Contract Manager | Ongoing training to new and existing staff Contract Manager Contract Manager | Ongoing training of Staff |
| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| Musculoskeletal Disorders (MSD) and injuries | Staff risk injury such as back problems if they try work in awkward positions or lift objects that are heavy and / or awkward to carry, such as cleaning machines or full mop buckets | Cleaning equipment provided and placed for ease of use at top and lower floor. Long handles mops, brushes and dusters are provided to reduce need to stretch and stoop. Staff instructed not to overfill buckets If lift available staff are to use, which reduces need for lifting the carrying | Provide ongoing training in manual handling | Contract Manager | On going | Continual training of staff |
| Working at height | Staff risk bruising and / or fractures if falling from a height | Staff are instructed not to work from step ladders. No standing of chairs policy. Staff are trained in safe system if work for cleaning stairs | Ongoing training | Contract Manager | On Going | Continual Training of Staff |
| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| Machine cleaning of floors | Staff and other risk injury from improper use of floor machines | Machines provided are the correct ones for the job. Cleaners are trained in the safe use of the machines provided. Machines are inspected by a competent person & maintained as nec. Clients are to instructed their staff not to use cleaning machinery if they are not qualified to do | Cleaners reminded not use if in any doubt of the safety of the machine Training in visual checks before commencement of use of the machine Ongoing training | Contract Manager | Ongoing Training | Implemented 03/01/2018 Implemented 03/01/2019 |

| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| --- | --- | --- | --- | --- | --- | --- |
| Lone working | Staff may suffer sudden illness / accident whilst working alone | Staff details to be kept up to date. Family member to notify Manager if cleaner not returned home at specified time Contract Manager or Manager on call to attend site to check on staff All staff instructed on safe locking up procedures if they are the last to leave the building or area being cleaned | Implement signing in / out books for site staff | Contract Manager | Immediate | System implemented |

| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| --- | --- | --- | --- | --- | --- | --- |
| Electrical Access and Egress | Staff risk electric shock or burns from faulty equipment Front entrance and emergency exits on each floor | Staff trained to check visually for damage to plugs, cables and on / off switches before using cleaning machines. If defect is noted the machine is taken out of use. Staff trained not to use water near electrical sockets or appliances Exit could be dimly lit during evening. Caution to be taken when approaching exit. Emergency lighting is adequate | Contract Manger to check plugs, cables and switches of cleaning machine PATT testing on machines when applicable Site induction for all members of staff. Staff to note all exits and how to raise the alarm in case of incident | Contract Manager Contract Manager | Start of contract |  |

| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| --- | --- | --- | --- | --- | --- | --- |
| Pregnancy | Staff may suffer sudden illness / accident whilst working alone Manual Handling | Staff details to be kept up to date. Managers numbers issued Pregnancy Risk assessment to be completed in person or across the phone. Contract Manager or Manager on call to attend site to check on staff (RE-LONE WORKING) All staff instructed on safe locking up procedures if they are the last to leave the building or area being cleaned | Implement clocking in and out procedures. / checked on by manager every hour. | Contract Manager | Immediate | System implemented |

| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| --- | --- | --- | --- | --- | --- | --- |
| Spread of COVID-19 | Staff/Client may suffer sudden illness Increased risk of spread of Covid-19 | Make extra PPE available to staff/ Disinfectant chemicals introduced. All touch points cleaned Daily. Hand Washing - Hand washing facilities with soap and hot water in place. - Stringent hand washing taking place. - Paper towels/hand dyers for drying of hands - See hand washing guidance. - https://www.nhs.uk/live-well/healthy-body/best-way-to-wash-your-hands/ - Gel sanitisers in any area where washing facilities not readily available | Employees to be reminded on a regular basis to wash their hands for 20 seconds with warm water and soap and the importance of proper drying. Also reminded to catch coughs and sneezes in tissues – Follow Catch it, Bin it, kill it and to avoid touching face, eyes, nose or mouth with unclean hands. Tissues will be made available throughout the workplace. To help reduce the spread of coronavirus (COVID-19) reminding everyone of the public health advice - https://www.publichealth.hscni.net/news/covid-19-coronavirus Posters, leaflets and other materials are available for display. https://www.gov.uk/government/publications/guidance-to-employers-and-businesses-about-covid-19 Rigorous checks will be carried out by line managers to ensure that the necessary procedures are being followed. Staff to be reminded on a daily basis of the importance of social distancing both in the workplace and outside of it. Management checks to ensure this is adhered to. Staff to be reminded that wearing of gloves is not a substitute for good hand washing. Face shields distributed for all staff. | Contract Manager | Immediate | System implemented |

| What are the hazards | Who might be harmed and how | Control measures | What further action is necessary | Action by who | Action by when | Completed |
| --- | --- | --- | --- | --- | --- | --- |
| Work vehicles How we drive Maintenance of Vehicles Bad weather causing dangerous roads. (bad visibility, Snow, heavy Rain Ice or Frost) Loading and unloading Vehicles(see manual handling) | Drivers / owners Others on the road. Pedestrians People being run over or struck by vehicle People being struck by something falling from a vehicles People falling from vehicles Vehicles overturning. | Check all users driving licences for points that Drive to and from work. Check awareness Work vans only to be used for transport to and from work. Instruct employees to take care while driving. Instruct Employees to follow the speed limits at all times. Take safer route if necessary. Yearly service on commercial Vans. Employees to check their vehicles before driving. Regular checks by HQ Introduce new vehicles for old ones Monitor driving routes. | Monitor and control the risks. | All | Immediate | System implemented |

Accident Log - Reported to Manager and Logged at HQ

Cleaners cupboard to be kept locked at all times-Client/Cleaner has the key

Cleaners to sign in and out / phone/App clocking supplied

Signed Management …………………

COSHH Sheets

- Terra Floor Cleaner

- Hand safe

- Freshnit

- Scaleit (Deep Cleans)

- Jangro Thick Bleach

- Hospec Toilet Cleaner

- Jangro Toilet Cleaner

- Jangro Lemon Floor Gel

- Lemon Multi Surface

- Jangro Bactericidal Hand Soap

COSHH Assessments

- Freshnit

- Scaleit (Deep Cleans)

- Jangro Thick Bleach

- Jangro Toilet Cleaner

- Jangro Lemon Floor Gel

- Terra Floor Ultra

| COSHH Assessment Form |  |  |  |  |
| --- | --- | --- | --- | --- |

| General Information |  |  |  |
| --- | --- | --- | --- |
| Substance Name | Freshnit | Assessor | Daniel Gibson |
| Process / Activity |  | Date | 02/02/22 |
| Location | All Sites | Sheet number | 001 |

| Substance Information |  |
| --- | --- |
| Workplace Exposure Limits | No Longer than 1 hour at a time |
| Hazard Statements / Risk Phrases Cautionary Statement ( R / H ) | Danger |
| Precautionary Statements | Keep out of reach of children Wear eye protection protective gloves Wash hand thoroughly after handling IF ON SKIN wash with plenty water If irritation or rash occurs get medical advice. If in Eye Rinse cautiously with water several Minutes Remove contact lenses if present and easy to do, continue rinsing If eye irritation persists, get medical Advice Avoid release into the environment Take off contaminated clothing. |
| Composition | Liquid |
| Quantity used / exposed to | Small. |

| Hazard Pictograms – select all that apply to the work activity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Health hazard | Toxic | Corrosive | Harmful/ Irritant | Flammable | Oxidising | Explosive | Compressed gas | Harmful to the environment |
|  |  | X | X |  |  |  |  | X |
| Other: |  |  |  |  |  |  |  |  |

| Activity information |  |
| --- | --- |
| Description of activity | Sprayed onto surfaces for disinfection |
| Persons exposed Directly | Cleaners |
| Persons exposed Indirectly | N/A |
| Frequency of use / exposure | Long-term exposure level 1Hour Short-term exposure level (15 mins): Frequency: Daily |
| Duration of the task or process | Various |
| Possible routes of entry | Inhalation (Absorption)Skin contact (Open wounds) Skin contact Eye contact Ingestion Injection |

| Dangerous Substances and Explosive Atmospheres (DSEAR) |  | No |
| --- | --- | --- |
| Are you carrying out an activity/chemical reaction that is at risk of thermal runaway or explosion? |  | no |
| Will the activity involve handling or storage of pyrophoric or unstable substances such as peroxide? |  | no |
| Will flammable vapours, solid particles, fibrous particles etc. capable of forming an explosive atmosphere be present in the working atmosphere? |  | no |

| Methods of Prevention or Control of Exposure |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Physical or Engineering Controls |  |  | Gloves to be worn at all times. Protective clothing. Safety Glasses |  |  |  |
| Administrative Controls |  |  | Introduction to chemical at induction (ongoing training) locked in cleaners’ cupboard/ labelled correctly in bottles for use. |  |  |  |
| Storage Requirements |  |  | Chemical to be locked in cleaner’s cupboard in 5ltr containers, to be decanted into 750ml bottles for use. All chemicals to be kept separately from others / keep container tightly closed. |  |  |  |
| Transport of the hazardous substance |  |  | Delivery of products as and when required. |  |  |  |
| Emergency procedures |  |  | Please see Health and safety at work follow spillage precautions. |  |  |  |
| Disposal Procedures Risk Phrases Cautionary Statement ( R / H ) |  |  | Avoid release into the environment. Do not discharge into drains or rivers. |  |  |  |
| PPE/RPE Required |  |  |  |  |  |  |
| Hand Protection | Dust Mask | Respirator* |  | Protective clothing | Eye protection | Other ? |
| x |  |  |  | x | x |  |
| Details/comments N/A |  |  |  |  |  |  |

| Approval |  |
| --- | --- |
| I confirm that this is a suitable and sufficient COSHH assessment for the above-described work activity |  |
| Assessor Name | Daniel Gibson |
| Signature |  |
| Date | 1st September 2023 |
| Risk Estimation | Low |

| COSHH Assessment Form |  |  |  |  |
| --- | --- | --- | --- | --- |

| General Information |  |  |  |
| --- | --- | --- | --- |
| Substance Name | SCALEIT | Assessor | D.GIBSON |
| Process / Activity |  | Date | 11/2/22 |
| Location | ALL SITES | Sheet number | 002 |

| Substance Information |  |
| --- | --- |
| Workplace Exposure Limits | No Longer than 1 hour at a time |
| Hazard Statements / Risk Phrases Cautionary Statement ( R / H ) | Danger |
| Precautionary Statements | Keep out of the reach of Children Wear protective clothes If swallowed rinse mouth does NOT induce vomiting Get immediate medical advice IF on skin Rinse Skin with Water If skin irritation occurs get medical advice/ attention If in Eyes rinse cautiously with water for several minutes If eye irritation continues get medical attention |
| Composition | Liquid |
| Quantity used / exposed to | Small |

| Hazard Pictograms – select all that apply to the work activity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Health hazard | Toxic | Corrosive | Harmful/ Irritant | Flammable | Oxidising | Explosive | Compressed gas | Harmful to the environment |
|  |  | X |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |  |  |

| Activity information |  |
| --- | --- |
| Description of activity | Used to remove scale from taps or surfaces |
| Persons exposed Directly | Cleaners |
| Persons exposed Indirectly | N/A |
| Frequency of use / exposure | Long-term exposure level 1 Hour Short-term exposure level (15 mins) Monthly |
| Duration of the task or process | Various |
| Possible routes of entry | Inhalation (Absorption)Skin contact (Open wounds) Skin contact Eye contact Ingestion Injection |

| Dangerous Substances and Explosive Atmospheres (DSEAR) | Yes | No |
| --- | --- | --- |
| Are you carrying out an activity/chemical reaction that is at risk of thermal runaway or explosion? |  | No |
| Will the activity involve handling or storage of pyrophoric or unstable substances such as peroxide? |  | No |
| Will flammable vapours, solid particles, fibrous particles etc. capable of forming an explosive atmosphere be present in the working atmosphere? |  | No |

| Methods of Prevention or Control of Exposure |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Physical or Engineering Controls |  |  | Gloves to be worn at all times. Protective clothing. Safety Glasses |  |  |  |
| Administrative Controls |  |  | Introduction to chemical at induction (ongoing training) locked in cleaners’ cupboard/ labelled correctly in bottles for use. |  |  |  |
| Storage Requirements |  |  | Chemical to be locked in cleaner’s cupboard in 5ltr containers, to be decanted into 750ml bottles for use. All chemicals to be kept separately from others / keep container tightly closed. |  |  |  |
| Transport of the hazardous substance |  |  | Delivery of products as and when required. |  |  |  |
| Emergency procedures |  |  | Please see Health and safety at work follow spillage precautions. |  |  |  |
| Disposal Procedures Risk Phrases Cautionary Statement ( R / H ) |  |  | Avoid release into the environment. Do not discharge into drains or rivers. |  |  |  |
| PPE/RPE Required |  |  |  |  |  |  |
| Hand Protection | Dust Mask | Respirator* |  | Protective clothing | Eye protection | Other ? |
| x |  |  |  | x | x |  |
| Details/comments * Face fit testing may be required |  |  |  |  |  |  |

| Approval |  |
| --- | --- |
| I confirm that this is a suitable and sufficient COSHH assessment for the above-described work activity |  |
| Assessor Name | D.GIBSON |
| Signature |  |
| Date | 1st September 2023 |
| Risk Estimation | Low |

| COSHH Assessment Form |  |  |  |  |
| --- | --- | --- | --- | --- |

| General Information |  |  |  |
| --- | --- | --- | --- |
| Substance Name | JANGRO THICK BLEACH 750ml | Assessor | D.GIBSON |
| Process / Activity |  | Date | 11/2/22 |
| Location | ALL SITES | Sheet number | 002 |

| Substance Information |  |
| --- | --- |
| Workplace Exposure Limits | No Longer than 1 hour at a time |
| Hazard Statements / Risk Phrases Cautionary Statement ( R / H ) | Danger Contact with Acids liberates toxic Gas Causes Burns Risk of serious damage to eyes Very toxic to aquatic Organisms |
| Precautionary Statements | Keep out of the reach of Children Wear protective clothes If swallowed rinse mouth does NOT induce vomiting Immediately call Doctor/ emergency services Get immediate medical advice IF on skin Rinse Skin with Water If skin irritation occurs get medical advice/ attention If in Eyes rinse cautiously with water for several minutes If eye irritation continues get medical attention |
| Composition | Liquid |
| Quantity used / exposed to | Small |

| Hazard Pictograms – select all that apply to the work activity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Health hazard | Toxic | Corrosive | Harmful/ Irritant | Flammable | Oxidising | Explosive | Compressed gas | Harmful to the environment |
|  |  | X |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |  |  |

| Activity information |  |
| --- | --- |
| Description of activity | Used to Disinfect and clean surfaces/Floors |
| Persons exposed Directly | Cleaners |
| Persons exposed Indirectly | N/A |
| Frequency of use / exposure | Long-term exposure level 1 Hour Short-term exposure level (15 mins) Monthly |
| Duration of the task or process | Various |
| Possible routes of entry | Inhalation (Absorption)Skin contact (Open wounds) Skin contact Eye contact Ingestion Injection |

| Dangerous Substances and Explosive Atmospheres (DSEAR) | Yes | No |
| --- | --- | --- |
| Are you carrying out an activity/chemical reaction that is at risk of thermal runaway or explosion? |  | No |
| Will the activity involve handling or storage of pyrophoric or unstable substances such as peroxide? |  | No |
| Will flammable vapours, solid particles, fibrous particles etc. capable of forming an explosive atmosphere be present in the working atmosphere? |  | No |

| Methods of Prevention or Control of Exposure |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Physical or Engineering Controls |  |  | Gloves to be worn at all times. Protective clothing. Safety Glasses |  |  |  |
| Administrative Controls |  |  | Introduction to chemical at induction (ongoing training) locked in cleaners’ cupboard/ labelled correctly in bottles for use. |  |  |  |
| Storage Requirements |  |  | Chemical to be locked in cleaner’s cupboard 750ml containers. All chemicals to be kept separately from others / keep container tightly closed. |  |  |  |
| Transport of the hazardous substance |  |  | Delivery of products as and when required. |  |  |  |
| Emergency procedures |  |  | Please see Health and safety at work follow spillage precautions. |  |  |  |
| Disposal Procedures Risk Phrases Cautionary Statement ( R / H ) |  |  | Avoid release into the environment. Do not discharge into drains or rivers. |  |  |  |
| PPE/RPE Required |  |  |  |  |  |  |
| Hand Protection | Dust Mask | Respirator* |  | Protective clothing | Eye protection | Other ? |
| x |  |  |  | x | x |  |
| Details/comments * Face fit testing may be required |  |  |  |  |  |  |

| Approval |  |
| --- | --- |
| I confirm that this is a suitable and sufficient COSHH assessment for the above-described work activity |  |
| Assessor Name | D.GIBSON |
| Signature |  |
| Date | 1st September 2023 |
| Risk Estimation | Low |

| COSHH Assessment Form |  |  |  |  |
| --- | --- | --- | --- | --- |

| General Information |  |  |  |
| --- | --- | --- | --- |
| Substance Name | Jangro Perfumed Toilet Cleaner | Assessor | D.GIBSON |
| Process / Activity |  | Date | 11/2/22 |
| Location | ALL SITES | Sheet number | 004 |

| Substance Information |  |
| --- | --- |
| Workplace Exposure Limits | No Longer than 1 hour at a time |
| Hazard Statements / Risk Phrases Cautionary Statement ( R / H ) | Danger May produced an allergic reaction Causes serious eye damage Very toxic to Aquatic life Causes skin irritation Harmful if swallowed |
| Precautionary Statements | Keep out of the reach of Children Wear protective clothes If swallowed rinse mouth does NOT induce vomiting Get immediate medical advice IF on skin Rinse Skin with Water If skin irritation occurs get medical advice/ attention If in Eyes rinse cautiously with water for several minutes If eye irritation continues get medical attention |
| Composition | Liquid |
| Quantity used / exposed to | Small |

| Hazard Pictograms – select all that apply to the work activity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Health hazard | Toxic | Corrosive | Harmful/ Irritant | Flammable | Oxidising | Explosive | Compressed gas | Harmful to the environment |
|  |  | X |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |  |  |

| Activity information |  |
| --- | --- |
| Description of activity | Washroom cleaning |
| Persons exposed Directly | Cleaners |
| Persons exposed Indirectly | N/A |
| Frequency of use / exposure | Long-term exposure level 1 Hour Short-term exposure level (15 mins) Monthly |
| Duration of the task or process | Various |
| Possible routes of entry | Inhalation (Absorption)Skin contact (Open wounds) Skin contact Eye contact Ingestion Injection |

| Dangerous Substances and Explosive Atmospheres (DSEAR) | Yes | No |
| --- | --- | --- |
| Are you carrying out an activity/chemical reaction that is at risk of thermal runaway or explosion? |  | No |
| Will the activity involve handling or storage of pyrophoric or unstable substances such as peroxide? |  | No |
| Will flammable vapours, solid particles, fibrous particles etc. capable of forming an explosive atmosphere be present in the working atmosphere? |  | No |

| Methods of Prevention or Control of Exposure |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Physical or Engineering Controls |  |  | Gloves to be worn at all times. Protective clothing. Safety Glasses |  |  |  |
| Administrative Controls |  |  | Introduction to chemical at induction (ongoing training) locked in cleaners’ cupboard/ labelled correctly in bottles for use. |  |  |  |
| Storage Requirements |  |  | Chemical to be locked in cleaner’s cupboard 750ml containers. All chemicals to be kept separately from others / keep container tightly closed. |  |  |  |
| Transport of the hazardous substance |  |  | Delivery of products as and when required. |  |  |  |
| Emergency procedures |  |  | Please see Health and safety at work follow spillage precautions. |  |  |  |
| Disposal Procedures Risk Phrases Cautionary Statement ( R / H ) |  |  | Avoid release into the environment. |  |  |  |
| PPE/RPE Required |  |  |  |  |  |  |
| Hand Protection | Dust Mask | Respirator* |  | Protective clothing | Eye protection | Other ? |
| x |  |  |  | x | x |  |
| Details/comments * Face fit testing may be required |  |  |  |  |  |  |

| Approval |  |
| --- | --- |
| I confirm that this is a suitable and sufficient COSHH assessment for the above-described work activity |  |
| Assessor Name | D.GIBSON |
| Signature |  |
| Date | 1st September 2023 |
| Risk Estimation | Low |

| COSHH Assessment Form |  |  |  |  |
| --- | --- | --- | --- | --- |

| General Information |  |  |  |
| --- | --- | --- | --- |
| Substance Name | Jangro Lemon Floor Gel | Assessor | D.GIBSON |
| Process / Activity |  | Date | 11/2/22 |
| Location | ALL SITES | Sheet number | 005 |

| Substance Information |  |
| --- | --- |
| Workplace Exposure Limits | No Longer than 1 hour at a time |
| Hazard Statements / Risk Phrases Cautionary Statement ( R / H ) | Danger May produced an allergic reaction Causes serious eye damage Very toxic to Aquatic life Causes skin irritation Harmful if swallowed |
| Precautionary Statements | Keep out of the reach of Children Wear protective clothes If swallowed rinse mouth does NOT induce vomiting Get immediate medical advice IF on skin Rinse Skin with Water If skin irritation occurs get medical advice/ attention If in Eyes rinse cautiously with water for several minutes If eye irritation continues get medical attention |
| Composition | Liquid |
| Quantity used / exposed to | Small |

| Hazard Pictograms – select all that apply to the work activity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Health hazard | Toxic | Corrosive | Harmful/ Irritant | Flammable | Oxidising | Explosive | Compressed gas | Harmful to the environment |
|  |  |  | X |  |  |  |  |  |
| Other: |  |  |  |  |  |  |  |  |

| Activity information |  |
| --- | --- |
| Description of activity | Floor cleaning |
| Persons exposed Directly | Cleaners |
| Persons exposed Indirectly | N/A |
| Frequency of use / exposure | Long-term exposure level 1 Hour Short-term exposure level (15 mins) Monthly |
| Duration of the task or process | Various |
| Possible routes of entry | Inhalation (Absorption)Skin contact (Open wounds) Skin contact Eye contact Ingestion Injection |

| Dangerous Substances and Explosive Atmospheres (DSEAR) | Yes | No |
| --- | --- | --- |
| Are you carrying out an activity/chemical reaction that is at risk of thermal runaway or explosion? |  | No |
| Will the activity involve handling or storage of pyrophoric or unstable substances such as peroxide? |  | No |
| Will flammable vapours, solid particles, fibrous particles etc. capable of forming an explosive atmosphere be present in the working atmosphere? |  | No |

| Methods of Prevention or Control of Exposure |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Physical or Engineering Controls |  |  | Gloves to be worn at all times. Protective clothing. Safety Glasses |  |  |  |
| Administrative Controls |  |  | Introduction to chemical at induction (ongoing training) locked in cleaners’ cupboard/ labelled correctly in bottles for use. |  |  |  |
| Storage Requirements |  |  | Chemical to be locked in cleaner’s cupboard 5Ltr containers. All chemicals to be kept separately from others / keep container tightly closed. |  |  |  |
| Transport of the hazardous substance |  |  | Delivery of products as and when required. |  |  |  |
| Emergency procedures |  |  | Please see Health and safety at work follow spillage precautions. |  |  |  |
| Disposal Procedures Risk Phrases Cautionary Statement ( R / H ) |  |  | Avoid release into the environment. |  |  |  |
| PPE/RPE Required |  |  |  |  |  |  |
| Hand Protection | Dust Mask | Respirator* |  | Protective clothing | Eye protection | Other ? |
| x |  |  |  | x | x |  |
| Details/comments * Face fit testing may be required |  |  |  |  |  |  |

| Approval |  |
| --- | --- |
| I confirm that this is a suitable and sufficient COSHH assessment for the above-described work activity |  |
| Assessor Name | D.GIBSON |
| Signature |  |
| Date | 1st September 2023 |
| Risk Estimation | Low |